Data solar field Almeria :

- HTF test loop erected in 1997

- Oriented East-West

- 18 bar maximum for HTF

- HTF : Syltherm 800

- Facility include :

  - 1m³ expansion tank (with nitrogen inertization)

  - Oil sump tank

  - Oil cooler (max 400 kWth)

  - Centrifugal pump (8.3 liter/sec max)

  - Electric oil heater

- Data Solar field :

  - 75m long

  - 4 Eurotrough first generation/2 second generation

  - 6 modules 12-m long  72 m

  - 18 receiver tube

  - Net aperture : 409.9m²

  - Absorber tube : UVAC (Universal Vacuum Collector, Solel)

    - UVAC properties :

      - 70 mm OD stainless tube + coating

      - Tube wall thickness = 2 mm

      - 115 mm OD antireflective evacuated tube

      - Glass wall thickness = 3 mm

      - 0.013 Pa vaacum (avoid convection)

      - Glass envelope transmittance : 0.96 (0.967 according rioglass website)

      - Coating absorptance : 0.95-96

      - Coating emmittance 0.15 or 0.091 at 400°C

    - There are already correlations for Q_dot_loss/m for UVAC2 (see Burkholder) and UVAC3

    - More technical info on http://www.rioglass.com/?page_id=1925

Source : PSA website

UVAC 1 and UVAC2 curves

Source : http://www.nrel.gov/docs/fy07osti/41423.pdf

UVAC 2 curves

Source : burkholder (PTR testing)

UVAC3 and UVAC2 curves

Source: burkholder UVAC3 report

Source : Price

Source : lupfert