Data solar field Almeria :

- HTF test loop erected in 1997

- Oriented East-West

- 18 bar maximum for HTF

- HTF : Syltherm 800

- Facility include :

  - 1m³ expansion tank (with nitrogen inertization)

  - Oil sump tank

  - Oil cooler (max 400 kWth)

  - Centrifugal pump (8.3 liter/sec max)

  - Electric oil heater

- Data Solar field :

  - 75m long

  - 4 Eurotrough first generation/2 second generation

  - 6 modules 12-m long  72 m

  - 18 receiver tube

  - Net aperture : 409.9m²

  - Absorber tube : UVAC (Universal Vacuum Collector, Solel)

    - UVAC properties :

      - 70 mm OD stainless tube + coating

      - Tube wall thickness = 2 mm

      - 115 mm OD antireflective evacuated tube

      - Glass wall thickness = 3 mm

      - 0.013 Pa vaacum (avoid convection)

      - Glass envelope transmittance : 0.96 (0.967 according rioglass website)

      - Coating absorptance : 0.95-96

      - Coating emmittance 0.15 or 0.091 at 400°C

    - There are already correlations for Q_dot_loss/m for UVAC2 (see Burkholder) and UVAC3

    - More technical info on http://www.rioglass.com/?page_id=1925

    - Other info from Forristal

      - Glass absorptance = 0.02

      - Glass emissivity = 0.86

| Eps1 – HCE shadowing factor | [-] | 1 (or Forristal) |
| --- | --- | --- |
| Eps2 – Tracking error factor | [-] | 1 (0.98 Valenzuela ppt) |
| Eps3 – Geometry error factor | [-] | 1 (or Forristal) |
| Eps4 – Dirt on Mirrors factor | [-] | 1 (or Forristal) |
| Eps5 – Dirt on HCE factor | [-] | 1 (or Forristal) |
| Eps6 – Unaccounted error factor | [-] | To adjust (1 at beginning) |
| Rho_cl – mirror reflectivity | [-] | 0.94 ou 0.931 (Lupfert) |
| Tau_g – glass transmittance | [-] | 0.92 (From Pyrex datasheet, but higher values up to 0.96 achieved in other HCE) |
| Alpha_g – Glass absorptance | [-] | 0.02 (Forristal) |
| Eta_opt_0 – Optimal optical efficiency | [-] | 0.684 (Sallaberry) |
| Eps_g – Glass emissivity | [-] | 0.86 (Forristal) |
| Alpha_t – tube absorptance | [-] | 0.7986 (recalculated) |
| Eps_t – Tube emissivity | [-] | ? no idea …. |
| a_K (coeff IAM theta) | [-] | +- 4.11e-3 (Sallaberry) |
| b_K (coeff IAM theta²) | [-] | +- 5.513e-5 |
| L – length of one collector | [m] | 71.16 (Sallaberry = 409.9/5.76) 71.88 (Lüpfert = 6*11.98) |
| A_p – aperture of the parabola | [m] | 5.76 |
| Dext_g – external glass diameter | [m] | 0.12 (Javier) |
| th_g – glass wall thickness | [m] | 0.0025 (Javier) |
| Rho_g – glass density | [kg/m³] | 2230 (from Pyrex datasheet http://www.qvf.com/glass-equipment/borosilicate-glass/physical-properties.html ) |
| Cp_g – glass specific heat capacity | [J/kg.K] | 900 (from Pyrex data http://www.qvf.com/glass-equipment/borosilicate-glass/physical-properties.html ) |
| Lambda_g – glass thermal conductivity | [W/m.K] | 1.14 (from Pyrex datasheet) |
| Dext_t – tube external diameter | [m] | 0.07 (Javier) |
| th_t – tube wall thickness | [m] | 0.002 (Javier) |
| Rho_t – tube density | [kg/m³] | 7990 (Javier) |
| Cp_t – tube specific heat capacity | [J/kg.K] |  |
| Lambda_t – tube thermal conducitvity | [W/m.K] |  |
| P_atm – Atmospheric pressure | [Pa] | 1e5 |
| K_air – thermal air conductivity | [W/m.K] | 0.025 |
| Rho_air – air density | [kg/m³] | 1.17 |
| Mu_air – air dynamic viscosity |  |  |
| Pvaccum – vaccum pressure | [Pa] | <113 Pa (1torr) |
| Gamma - ratio of specific heats for the annulus gas | [-] | 1.39 |
| Delta – Molecular diameter for the annulus gas | [cm] | 3.53e-8 (Forristal) |
| BB – interaction coefficient | [-] | 1.571 (Forristal) |
| K_st – thermal conductivity at standard pressure and temperatur | [W/m.K] | 0.02551 (Forristal) |

http://www.azom.com/article.aspx?ArticleID=4765

Source : PSA website

UVAC 1 and UVAC2 curves

Source : http://www.nrel.gov/docs/fy07osti/41423.pdf

UVAC 2 curves

Source : burkholder (PTR testing)

UVAC3 and UVAC2 curves

Source: burkholder UVAC3 report

Source : Price

Source : lupfert